CPE301 – SPRING 2019

# Design Assignment X

Student Name: Jett Guerrero

Student #: 5002377713

Student Email: guerrj1@unlv.nevada.edu

Primary Github Address: https://github.com/guerrj1/Submission_DA.git

Directory: https://github.com/guerrj1/Submission_DA/tree/master/DA2A

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

-ATmega328P

-Arduino Multi-Function Shield

-Micro-USB cable

PB2 and PC2 PINS were used on the Atmega328P and Arduino Shield

- DEVELOPED CODE OF TASK 1

Task 1 Assembly Code

;DA2AT1A

.INCLUDE "M328pDEF.INC"

.CSEG

.org 0x0000

SBI DDRB, 5          ;makes PB5 an output

SBI PORTB,5          ;turns off LED for PB5

SBI DDRB, 2          ;makes PB2 an output

LDI R20,5            ;set clock prescaler to 1024

STS TCCR1B,R20

begin:

LDI R20,0x00         ;resetting the counter to 0

;ON

SBI PORTB, 2         ;sets LED PB2 on

STS TCNT1H,R20

STS TCNT1L,R20

RCALL delay_on       ;calling timer to wait for 0.435 sec

;OFF

CBI PORTB, 2         ;clears LED PB2 off

STS TCNT1H,R20

STS TCNT1L,R20

RCALL delay_off      ;calling timer to wait for 0.29 sec

RJMP begin           ;repeat main loop

;Delay for LED on-----------

delay_on:

LDS R29, TCNT1H      ;loading upper bit of counter to R29

LDS R28, TCNT1L      ;loading lower bit of counter to R28

CPI R28,0x8C         ;comparing if lower 8 bits of timer is 0x8c - lower bits of 1A8C

BRSH body_on         ;if lower bits of timer have reached desired amount, check the upper bits

RJMP delay_on        ;otherwise, keep checking lower bits

body_on:

CPI R29,0x1A         ;check to see if upper timer bits have reached the desired value

BRLT delay_on        ;if not, recheck the lower bits

RET                  ;once the timer reached the desired value, toggle the LED

;Delay for LED off------------

delay_off:

LDS R29, TCNT1H      ;loading upper bit of counter to R29

LDS R28, TCNT1L      ;loading lower bit of counter to R28

CPI R28,0xB2         ;comparing if lower 8 bits of timer is 0xB2 - lower bits of 11B2

BRSH body_off        ;if lower bits of timer have reached desired amount, check the upper bits

RJMP delay_off       ;otherwise, keep checking lower bits

body_off:

CPI R29,0x11         ;check to see if upper timer bits have reached the desired value

BRLT delay_off       ;if not, recheck the lower bits

RET                  ;once the timer reached the desired value, toggle the LED

Task 1 C Code

//DA2AT1C.c

#include <avr/io.h>

#include <stdio.h>

int main(void)

{

DDRB = (1<<2);   //sets pb2 as an input

DDRB = (0<<5);   //sets pb5 as an input

TCCR1B = 5;      //sets the prescaler of 1024

while(1)

{

TCNT1 = 0;   //t count

while (TCNT1 != 4530) //while tcount is not equal to the calculated tcnt1

{}                    //creates the delay

PORTB ^= (1<<2);     //sets pb2 led to on

TCNT1 = 0;           //resets count to 0

while (TCNT1 != 6796)  //while tcount is not euqal to the calculated tcnt1 value

{}                     //set delay

PORTB ^= (1<<2);       //exclusive OR the previous value to toggle

}

return 0;

}

- DEVELOPED FOR TASK 2

Task 2 Assembly Code

; DA2AT2A.asm

.CSEG

.org	0x0000

SBI DDRB, 5             ;makes PB5 an output

SBI PORTB,5             ;turns of LED for PB5

CBI DDRC, 2             ;makes PC2 an input

LDI R17,0               ;used to set or reset PB5

SBI DDRB, 2             ;makes PB2 an output

LDI R20,5               ;set clock prescaler to 1024

STS TCCR1B, R20

check:

SBIC PINC, 2            ;skip next instruction if PC2 is clear

RJMP waiting            ;jumps to waiting label for waiting for a pressed input

CBI PORTB, 2            ;clears PB2

STS TCNT1H, R20

STS TCNT1L, R20

RCALL delay_on          ;jumping for LED on delay

RJMP check              ;jump back to check

;waiting for input from pushbutton

waiting:

SBI PORTB, 2            ;sets PB2 on

RJMP check              ;relative jump back to check label

;delay for LED on

delay_on:

LDS R29, TCNT1H         ;loading upper bit of counter to R29

LDS R28, TCNT1L         ;loading lower bit of counter to R28

CPI R28,0x4A            ;comparing if lower 8 bits of timer is 0x08

BRSH body_on            ;if lower bits of timer have reached desired amount, check the upper bits

RJMP delay_on           ;otherwise, keep checking lower bits

body_on:

CPI R29,0x4C            ;check to see if upper timer bits have reached the desired value

BRLT delay_on           ;if not, recheck the lower bits

RET                     ;once the timer reached the desired value, toggle the LED

Task 1 C Code

//DA2AT2C.c

#define F_CPU 16000000UL        //sets CPU clock speed

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRB |= (1<<2);                    //sets pb2 as input

PORTB |= (1<<2);                   //sets pb2 to high for LED

DDRC &= (0<<2);                    //sets PC2 to input

PORTC |= (0<<2);                   //sets PC2 to low 0

while (1)                          //while loop continuously checking

{

if (!(PINC & (1 << PINC2)))    //if pinc2 was pressed then released

{

PORTB &= ~(1<<2);          //then portb2 led is on

_delay_ms(1250);           //delays for 1.250 secs while on

}

else

{

PORTB |= (1<<2);           //toggles pb2 led

}

}

return 0;

}

- SCHEMATICS

Breadboard Schematic using Fritzing

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Task 1 Duty Cycle and Period Verification

60% Duty Cycle 434.95ms ≈ 0.435s                           Total Period 724.87ms ≈  0.725s

40% Duty Cycle 289.92ms ≈ 0.29s

Task 2 Duty Cycle and Period Verification

LED ON Delay for 1,250ms ≈ 1.250s

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task 1

LED OFF                                                                              LED ON

Task 2

LED OFF                                                                            LED Turns OFF After Push Button is Pressed

LED Stays ON for the 1.250s

- VIDEO LINKS OF EACH DEMO

Task 1 Assembly Demo

https://youtu.be/Jb8sLYPLfoo

Task 1 C Demo

https://youtu.be/pDwe_vSzwnU

Task 2 Assembly Demo

https://youtu.be/B7cBseegcOA

Task 2 C Demo

https://youtu.be/yB9F5vjMUPY

- GITHUB LINK OF THIS DA

https://github.com/guerrj1/Submission_DA/tree/master/DA2A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Jett Guerrero