CPE301 – SPRING 2019

# Design Assignment DA2B

Student Name: Jett Guerrero

Student #: 5002377713

Student Email: guerrj1@unlv.nevada.edu

Primary Github address: https://github.com/guerrj1/Submission_DA.git

Directory: DA2B – https://github.com/guerrj1/Submission_DA/tree/master/DA2B

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

-ATMega328p

-Arduino Shield

-Micro-USB Cable

-Male to Male Wire

- DEVELOPED CODE OF TASK 1 ASSEMBLY CODE

; DA2BT1.asm

.INCLUDE "M328PDEF.INC"

.org 0                 ;location reset

jmp main

.org 0x02              ;location for external interrupt 0

jmp ex0_isr            ;external interrupt label

main:

ldi r20, high(ramend)  ;loads the upper ram address

out sph, r20

ldi r20, low(ramend)   ;loads lower ram address

out spl, r20           ;initialize the stack

sts eicra, r20         ;external interrupt control register

sbi portd, 2           ;pull-up activated

sbi ddrb, 2            ;set portb2 as output

cbi portb, 2           ;clears portb2

sbi ddrb, 5            ;sets pb5 as output

sbi portb, 5           ;turns off pb5

ldi r20, (1<<int0)     ;enable INT0

out eimsk, r20         ;store EIMSK

LDI R20,5              ;set clock prescaler to 1024

STS TCCR1B, R20

sei                    ;enable interrupts

ex0_isr:               ;interrupt label

in r21, portb          ;portb value stored into r21

ldi r22, (1<<2)        ;sets a one in bit 2

eor r21, r22           ;toggles the led

out portb, r21         ;outputs the led

STS TCNT1H, R20

STS TCNT1L, R20

rcall delay_on         ;calls the delay to keep led on

reti

delay_on:

LDS R29, TCNT1H       ;loading upper bit of counter to R29

LDS R28, TCNT1L       ;loading lower bit of counter to R28

CPI R28,0x4A          ;comparing if lower 8 bits of timer is 0x08

BRSH body_on          ;if lower bits of timer have reached desired amount, check the upper bits

RJMP delay_on         ;otherwise, keep checking lower bits

body_on:

CPI R29,0x4C          ;check to see if upper timer bits have reached the desired value

BRLT delay_on         ;if not, recheck the lower bits

RET                   ;once the timer reached the desired value, toggle the LED

- DEVELOPED CODE FOR TASK 1 C CODE

//DA2BT1C.c

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

int main(void)

{

DDRB |= (1<<2);    //sets pb2 as an output

DDRB |= (1<<5);    //sets pb5 as an output

PORTB |= (1<<2);   //sets pb2 led

PORTB |= (1<<5);   //sets pb5 led off

DDRC &= (0<<2);    //for push button output

PORTC |= (0<<2);   //for push button

PORTD |= (1<<2);   // enable PORTD.2 pin pull up resistor

EIMSK = (1<<INT0); //enable external interrupt 0

EICRA = 0x02;      //interrupt on falling edge

sei();

while(1)           //while running

{}

}

ISR(INT0_vect)

{

PORTB ^= (1<<2); //toggle pb2 led

_delay_ms(1250);   //keep led on for 1250ms

}

- SCHEMATICS

In this schematic, we are using PD2 for INT0. To activate PD2, we can use a pushbutton to trigger a 0 which will cause an interrupt in the system.  The schematic on the right shows a simple schematic of the logic.

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Assembly Processor Status Simulation for 1.25 sec Delay

C Code Processor Status Simulation for 1.25 sec Delay

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

LED is initially off. Turns on after pressing the push button and stay on for 1.25 secs.

- VIDEO LINKS OF EACH DEMO

Assembly Code Demo

https://youtu.be/neLtCoso4XY

C Code Demo

https://youtu.be/PS8ZCJxHJcA

- GITHUB LINK OF THIS DA

https://github.com/guerrj1/Submission_DA/tree/master/DA2B

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Jett Guerrero